FWC IITH INTERNSHIP

DIGITAL – DESIGN

AVR – GCC:

AVR-GCC is a compiler that is part of the AVR-GNU toolchain, and it is specifically designed for Atmel AVR microcontrollers. AVR microcontrollers are widely used in embedded systems and digital design applications due to their low power consumption, simplicity, and cost-effectiveness. AVR-GCC is based on the GCC (GNU Compiler Collection), a popular open-source compiler suite.

Here's a detailed explanation of AVR-GCC in terms of digital design:

1. **Compiler Overview:**

- **Language Support:** AVR-GCC supports the C and C++ programming languages, making it suitable for a wide range of developers. C is commonly used in embedded systems due to its low-level features and efficiency.

- **Cross-Compilation:** AVR-GCC is a cross-compiler, meaning it runs on one architecture (e.g., x86) but generates code for another architecture (e.g., AVR). This allows developers to write code on a more powerful machine and then compile it for the AVR microcontroller.

2. **AVR Microcontrollers:**

- **Architecture:** AVR microcontrollers are based on the Harvard architecture, featuring separate program and data memories. They typically have a small amount of RAM, Flash memory for program storage, EEPROM for non-volatile data storage, and various peripherals such as timers, UART, GPIO, etc.

- **Instruction Set:** AVR microcontrollers use a reduced instruction set computing (RISC) architecture, which simplifies the design and enhances performance. The instruction set is relatively small but efficient, making it well-suited for embedded systems.

3. **AVR-GCC Features:**

- **Optimization:** AVR-GCC provides optimization options to enhance code performance and reduce code size. These optimizations can be crucial in embedded systems with limited resources.

- **Library Support:** AVR-GCC includes libraries that provide ready-to-use functions for common tasks. These libraries are designed to work seamlessly with AVR microcontrollers, offering functions for tasks such as GPIO manipulation, communication protocols (e.g., SPI, I2C), and more.

- **Inline Assembly:** Developers can use inline assembly within their C or C++ code to write low-level instructions directly, providing fine-grained control over the microcontroller's resources.

4. **Programming Workflow:**

- **Code Development:** Developers write their code using a text editor or an integrated development environment (IDE). The code typically includes low-level operations and interacts with the AVR peripherals.

- **Compilation:** The code is then compiled using AVR-GCC, which translates the high-level code into machine code suitable for the AVR microcontroller.

- **Linking:** The compiled code is linked with any necessary libraries to create the final executable file.

- **Flashing:** The resulting binary file is then flashed onto the AVR microcontroller's Flash memory using a programmer, enabling the microcontroller to execute the program.

5. **Community and Documentation:**

- AVR-GCC benefits from the open-source community and has extensive documentation available. This allows developers to find solutions to common issues, learn best practices, and contribute to the improvement of the toolchain.

In summary, AVR-GCC is a powerful tool in the digital design landscape, enabling developers to write efficient code for AVR microcontrollers. Its cross-compilation capability, optimization features, and extensive community support make it a valuable tool for embedded systems and other digital design applications.

CODE:

C++ Code [General]:

// define the pin number for the LED

#define LED_PIN 13

// define the time interval for each state in milliseconds

#define TIME_INTERVAL 1000

void setup() {

// initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

}

void loop() {

// turn the LED on for state 00

digitalWrite(LED_PIN, HIGH);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED off for state 10

digitalWrite(LED_PIN, LOW);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED on for state 01

digitalWrite(LED_PIN, HIGH);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED off for state 11

digitalWrite(LED_PIN, LOW);

// wait for one second

delay(TIME_INTERVAL);

}

AVR-GCC Libraries Included Code:

#include <avr/io.h>

#include <util/delay.h>

// Define the pin number for the LED

#define LED_PIN 13

// Define the time interval for each state in milliseconds

#define TIME_INTERVAL 1000

void setup() {

// Set the LED pin as an output

DDRB |= (1 << DDB5);

}

void loop() {

// Turn the LED on for state 00

PORTB |= (1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED off for state 10

PORTB &= ~(1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED on for state 01

PORTB |= (1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED off for state 11

PORTB &= ~(1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

}

int main(void) {

setup();

while (1) {

loop();

}

return 0;

}

EXPLANATION:

Both codes perform the same functionality of toggling an LED connected to a specific pin with a specified time interval between on and off states. The first code is written using the Arduino framework, which abstracts low-level details, while the second code is written in C++ with direct use of AVR-GCC libraries, providing more control over hardware.

Let's go through each code line by line:

### C++ Code [General]:

```cpp

// define the pin number for the LED

#define LED_PIN 13

// define the time interval for each state in milliseconds

#define TIME_INTERVAL 1000

void setup() {

// initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

}

```

1. **`#define LED_PIN 13`:** This line defines a symbolic name `LED_PIN` with a value of 13. This is typically used to make the code more readable by replacing the actual pin number with a meaningful name.

2. **`#define TIME_INTERVAL 1000`:** Similarly, this line defines a symbolic name `TIME_INTERVAL` with a value of 1000 (milliseconds). It represents the time interval for each state (on or off) of the LED.

3. **`void setup() {...}`:** The `setup` function is part of the Arduino framework. It is called once when the program starts and is used for initialization.

4. **`pinMode(LED_PIN, OUTPUT);`:** This line configures the specified pin (`LED_PIN`) as an output, indicating that it will be used to send signals (in this case, to control the LED).

```cpp

void loop() {

// turn the LED on for state 00

digitalWrite(LED_PIN, HIGH);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED off for state 10

digitalWrite(LED_PIN, LOW);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED on for state 01

digitalWrite(LED_PIN, HIGH);

// wait for one second

delay(TIME_INTERVAL);

// turn the LED off for state 11

digitalWrite(LED_PIN, LOW);

// wait for one second

delay(TIME_INTERVAL);

}

```

5. **`void loop() {...}`:** The `loop` function is also part of the Arduino framework and is called repeatedly after the `setup` function. It contains the main logic of the program.

6. **`digitalWrite(LED_PIN, HIGH);`:** This line sets the specified pin (`LED_PIN`) to HIGH, turning the LED on.

7. **`delay(TIME_INTERVAL);`:** This line pauses the execution for the specified time interval (`TIME_INTERVAL`) in milliseconds.

8. **`digitalWrite(LED_PIN, LOW);`:** This line sets the specified pin (`LED_PIN`) to LOW, turning the LED off.

9. **`delay(TIME_INTERVAL);`:** Another delay to wait before moving on to the next state.

The loop continues, toggling the LED on and off in a predefined pattern.

### AVR-GCC Libraries Included Code:

```cpp

#include <avr/io.h>

#include <util/delay.h>

// Define the pin number for the LED

#define LED_PIN 13

// Define the time interval for each state in milliseconds

#define TIME_INTERVAL 1000

void setup() {

// Set the LED pin as an output

DDRB |= (1 << DDB5);

}

```

1. **`#include <avr/io.h>` and `#include <util/delay.h>`:** These lines include the necessary AVR-GCC libraries for direct hardware manipulation and delay functions.

2. **`#define LED_PIN 13` and `#define TIME_INTERVAL 1000`:** Similar to the previous code, these lines define symbolic names for the LED pin and time interval.

3. **`void setup() {...}`:** The setup function in this code initializes the hardware.

4. **`DDRB |= (1 << DDB5);`:** This line sets the 6th bit (corresponding to pin B5) of the `DDRB` register to 1, configuring the specified pin as an output.

```cpp

void loop() {

// Turn the LED on for state 00

PORTB |= (1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED off for state 10

PORTB &= ~(1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED on for state 01

PORTB |= (1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

// Turn the LED off for state 11

PORTB &= ~(1 << PORTB5);

// Wait for one second

_delay_ms(TIME_INTERVAL);

}

```

5. **`void loop() {...}`:** Similar to the Arduino version, this loop function contains the main logic.

6. **`PORTB |= (1 << PORTB5);`:** This line sets the 6th bit of the `PORTB` register to 1, turning the LED on.

7. **`_delay_ms(TIME_INTERVAL);`:** This line provides a delay using the `_delay_ms` function for the specified time interval.

8. **`PORTB &= ~(1 << PORTB5);`:** This line clears the 6th bit of the `PORTB` register, turning the LED off.

9. **`_delay_ms(TIME_INTERVAL);`:** Another delay before moving to the next state.

The loop continues, toggling the LED on and off in the specified pattern.

### Additional Note for AVR-GCC Code:

The AVR-GCC code includes a `main` function that calls the `setup` function once and then enters an infinite loop calling the `loop` function. This is a more traditional structure for embedded systems programming using AVR-GCC, where you have more control over the initialization process. In contrast, the Arduino framework abstracts this structure away, making it simpler for beginners and allowing for rapid prototyping.

CIRCUIT CONNECTION:

For both C++ code (Arduino) and AVR-GCC code, the circuit connections are similar. They involve connecting an LED to a specific pin on the microcontroller and controlling its state using that pin. Below are the detailed circuit connections for both cases:

### C++ Code (Arduino):

1. Connect the positive (longer) leg of the LED to digital pin 13 on the Arduino board.

2. Connect the negative (shorter) leg of the LED to the ground (GND) pin on the Arduino board.

### AVR-GCC Libraries Included Code:

1. Connect the positive (longer) leg of the LED to digital pin 13 on the microcontroller.

- In AVR terms, this corresponds to pin B5 on the ATmega microcontroller.

2. Connect the negative (shorter) leg of the LED to the ground (GND) pin on the microcontroller.

### Common Points:

- Both codes use the same LED_PIN (pin 13) for controlling the LED.

- The TIME_INTERVAL defines the duration for which the LED is in each state (ON or OFF). It is set to 1000 milliseconds (1 second) in both cases.

- The setup function initializes the designated pin as an output.

Please note that in the AVR-GCC code, the `PORTB` register is used to control the state of pin B5, equivalent to digital pin 13 on an Arduino board. The `DDRB` register is used to set pin B5 as an output. The `_delay_ms` function from the `<util/delay.h>` library is used for creating delays.

Ensure that you have the LED connected correctly with the longer leg to the specified digital pin and the shorter leg to ground for the code to work as intended.

Circuit Connections Code – 1 :

Connect the positive (longer leg) of the LED to digital pin 13 (D13) on the Arduino Uno.

Connect the negative (shorter leg) of the LED to one of the ground (GND) pins on the Arduino Uno.

Circuit Connections Code – 2 :

Connect the positive (longer leg) of the LED to digital pin 13 (PB5) on the Arduino Uno.

Connect the negative (shorter leg) of the LED to one of the ground (GND) pins on the Arduino Uno.

WITH TWO LEDs:

C++ Code:

// Define the digital pins for the LEDs

const int ledPin1 = 2; // Digital pin 2 for LED 1

const int ledPin2 = 3; // Digital pin 3 for LED 2

void setup() {

// Set pin modes

pinMode(ledPin1, OUTPUT);

pinMode(ledPin2, OUTPUT);

}

void loop() {

// Sequence: 00

digitalWrite(ledPin1, LOW);

digitalWrite(ledPin2, LOW);

delay(1000); // 1 second delay

// Sequence: 10

digitalWrite(ledPin1, HIGH);

digitalWrite(ledPin2, LOW);

delay(1000); // 1 second delay

// Sequence: 01

digitalWrite(ledPin1, LOW);

digitalWrite(ledPin2, HIGH);

delay(1000); // 1 second delay

// Sequence: 11

digitalWrite(ledPin1, HIGH);

digitalWrite(ledPin2, HIGH);

delay(1000); // 1 second delay

// Sequence: 00 (Reset)

digitalWrite(ledPin1, LOW);

digitalWrite(ledPin2, LOW);

delay(1000); // 1 second delay

}

AVR-GCC Code:

#include <avr/io.h>

#include <util/delay.h>

// Define the pins for the LEDs

#define LED_PIN1 PB2 // Digital pin 2 for LED 1

#define LED_PIN2 PB3 // Digital pin 3 for LED 2

int main(void) {

// Set pins as output

DDRB |= (1 << LED_PIN1) | (1 << LED_PIN2);

while (1) {

// Sequence: 00

PORTB &= ~((1 << LED_PIN1) | (1 << LED_PIN2));

_delay_ms(1000); // 1 second delay

// Sequence: 10

PORTB |= (1 << LED_PIN1);

PORTB &= ~(1 << LED_PIN2);

_delay_ms(1000); // 1 second delay

// Sequence: 01

PORTB &= ~(1 << LED_PIN1);

PORTB |= (1 << LED_PIN2);

_delay_ms(1000); // 1 second delay

// Sequence: 11

PORTB |= (1 << LED_PIN1) | (1 << LED_PIN2);

_delay_ms(1000); // 1 second delay

// Sequence: 00 (Reset)

PORTB &= ~((1 << LED_PIN1) | (1 << LED_PIN2));

_delay_ms(1000); // 1 second delay

}

return 0;

}